# Optimal Analysis for Facility Configuration and Energy Management on Electric Light Commercial Vehicle Charging

Yanyan Tang

State Key Joint Laboratory of Environment Simulation and Pollution Control, School of Environment, Tsinghua University, Beijing, 100084, China

Email: yytang@mail.tsinghua.edu.cn

Qi Zhang (Corresponding author)

School of Economics and Management, China University of Petroleum-Beijing, Beijing, 102249, China

Academy of Chinese Energy Strategy, China University of Petroleum-Beijing, Beijing, 102249, China

Email: zhangqi56@tsinghua.org.cn

Zongguo Wen (Corresponding author)

State Key Joint Laboratory of Environment Simulation and Pollution Control, School of Environment, Tsinghua University, Beijing, 100084, China

Email: wenzg@tsinghua.edu.cn

Derek Bunn

Management Science and Operations, London Business School, London, NW1 4SA, UK

Email: dbunn@london.edu

Jesus Nieto Martin

Cable Energia, Carretera de la Exclusa 11, 41011 Sevilla, España

Email: jmartin@london.edu

# Highlights

- The enhanced benefits of public charging stations in four business models are compared.

- A two-stage model regarding the infrastructure investment and operational planning is formulated.

- The uncertainties of electricity tariff and solar generation are considered for more realistic simulations.

- The integrated business models are conducive to increasing the investment viability and reducing the carbon intensity.

# Abstract

Whilst the widespread adoption of electric vans is necessary to improve urban air quality and reduce carbon emissions, it is also self-evident that adequate charging stations are a precondition. However, the investment case for basic charging stations without public subsidies is challenging. In the context of a London case study, four business models are compared, which integrate solar power generation and new/second-life battery storage system with the basic charging facilities. Considering the uncertainties of electricity tariff and solar generation, the optimal infrastructure investment and operational planning has been formulated as a two-stage stochastic optimization model. The results show that: (i) in the integrated business models, the return on investment and charger installations could be increased by up to 5.39% and 17.06% respectively, and the carbon intensity could be reduced by up to 8.13%; (ii) the nondiscriminatory grant annualized as 50 £ is not sufficient, and a differentiated government subsidy policy may be more conducive to achieving a positive return on investment, such as 50 £ for fast chargers and 100 £ for rapid chargers; (iii) in the integrated business models, fast chargers undertake more vehicle-to-grid electricity exchange with the pattern adoption rate increased by up to 52.38%, while rapid chargers mainly ensure the timely charging completion with the usage frequency increased by up to 2.82%.

Keywords: Electric vans; Public charging station; Investment viability; Business models

# Introduction

Although commercial vehicles make up a slighter market share of urban transportation, they can emit around 30% higher carbon dioxide emissions per kilometer than passenger cars [1]. With the widespread policy ambitions for reducing oil dependency and combating climate change, Light Commercial Vehicles (LCVs), commonly referred as ‘vans’, will switch away from diesel and most likely into electric [2–4]. In 2019, new registrations of Electric Light Commercial Vehicles (ELCVs) rose by a quarter over the previous year, with the global stock reaching 378,000. Nevertheless, the combustion engine still dominates the LCV market. For instance, roughly 3300 electric vans were sold in UK in 2019, accounting for only 0.91% of overall sales [5].

There are a number of significant challenges which hold back the penetration of electric vans, such as the power grid reinforcement [6], sale price imparity[7], and especially range anxiety concerns [8].Moreover, there is a lack of charging infrastructure. The ratio of global EVs to publicly accessible chargers was approximately 9.51 in 2019, substantially worse than the recommended benchmark of 8 [9]. Another key issue is to establish a balance between speedy-yet-costly rapid charging and inexpensive-but-inconvenient fast charging, especially for electric vans which would want to reduce idle parking time [10]. In this context, the basic charging station projects have been financially unattractive, presenting payback time between 17 and 26.5 years [11]. In addition, the cost of a rapid charger (RC) is estimated to be almost an order of magnitude higher than that of a fast charger (FC) [12], making RCs not the preferred facility option. For instance, as shown in Figure 1, RCs took up only 7.41% of the total public charging devices in London [13].

Figure 1. Proportion of public rapid chargers in London

Currently, there are only a few targeted incentives aimed at developing charging infrastructure and they mainly take the form of general subsidies. The UK government, for example in 2020, provides basic funding of 350 £ for each charger [14]. However, with the technology readiness of end-user resources, such as solar panels and new, or second-life batteries, more business models are emerging, which may be more cost-effective compared with basic charging stations. Therefore, we seek to address the research questions of whether there are enhanced benefits with integrated energy resources, including photovoltaic (PV) and batteries, on the investment attractiveness of charging stations. In so doing, it is necessary to envisage a more complex optimization model, which integrates the infrastructure investment together with the consequent operational planning in the face of daily uncertainties.

# Background research

To clarify the contribution of this paper, previous research on charging stations will be discussed from three research streams: target customers, business models, and optimization method.

So far, the load curve of charging stations is mainly derived by taking passenger cars as the research object. According to a sample survey in six European countries, Pasaoglu et al. [15] observed that the passenger EVs were mainly used for short home-based trips with a daily mileage of 40-80 km. Furthermore, their average duration of parking time was estimated to be around 6 hours per day, indicating that a slow topping up would be relatively sufficient [16]. Other stochastic features were also examined, such as arrival time, departure time, initial state-of-charge (SOC) level and charging demand [17–19]. There are distinctive characteristics between passenger EVs and ELCVs. For instance, commercial cars generally conduct more long-distance trips. Baford et al. [20] and Christensen et al. [21] found that nearly 31% and 29% of ELCVs would perform more than 100 km per day in Denmark and Germany, respectively. In terms of charging behavior, it was found that a larger percentage of charging events took place at nighttime for private vehicles than the commercial vehicles [22]. Since the nature of the service business required punctuality, it was noted that commercial users were willing to detour up to 500 m for fast charging, which was 70% less than the distance for private users [23,24]. In summary, commercial vehicles are confronted with more uncertainties, such as the route selection [25], parking duration [24], parking times [26] and charging timing [22]. However, as an important participant in the public charging, the typical driving characteristics of ELCVs have not yet been fully considered, which promotes this study to make some contributions into this aspect.

As summarized in Table 1, the existing research mainly focusses on the impacts of several charging facilities and services in a particular business model. For instance, Neyestani et al. [17] found that the Vehicle-to-Grid (V2G) service could stabilize the operation of distribution network and enrich the revenue sources. The possibility of coordinating renewable generation to power charging stations was also investigated [27]. In terms of energy storage system, Figueiredo et al. [11] put forward that when its price dropped to a certain level in the future, the integration with solar parking lots would be economically feasible. In this context, the use of retired EV batteries appears to be potentially competitive. Apart from Tong et al. [28] and Han et al. [29], there has been relatively little research on their applications in charging stations. With evolution of various business models, it is necessary to fully analyze the enhanced benefits of different integrated facility combinations, so as to improve the financial viabilities of charging infrastructure. However, few studies have considered the renewable generation, new/second-life battery storage system, and fast/rapid chargers in the same framework. This study seeks to fill this research gap.

Table 1. Summary of the related research

| Research | Charging facilities and service involved in the public charging station |  |  |  |  |  | Fixed configuration? |  | Stochastic perspective? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FC | RC | PV | New battery | Second-life battery | V2G service | FC or RC | Other facilities |  |
| Schroeder &Traber [30] | √ | √ |  |  |  |  | Yes | Yes | No |
| Mouli et al. [31] | √ |  | √ |  |  | √ |  |  |  |
| Shafie-Khah et al. [18] | √ |  |  | √ |  | √ |  |  | Yes |
| Cardoso et al. [32], Neyestani et al. [17], Shafie-Khah et al. [19] | √ |  | √ | √ |  | √ |  |  |  |
| Mouli et al. [16], Figueiredo et al. [11], Novoa & Brouwer [27] |  |  |  |  |  |  |  |  | No |
| Tong et al. [28] | √ |  | √ |  | √ |  |  |  |  |
| Funke et al. [33], Muratori et al.[34] |  | √ |  |  |  |  |  |  |  |
| Muratori et al. [35] |  | √ | √ | √ |  |  |  |  |  |
| Han et al. [29] | √ | √ | √ |  | √ |  |  | No |  |
| Luo et al. [36] |  | √ | √ |  |  |  | No | No |  |

Different mathematical optimization problems have been proposed to minimize the total cost of public charging stations. As shown in Table 1, they are mainly based on the precondition that the facility configuration has been determined. For example, Figueiredo et al. [11] developed a genetic algorithm to optimize charging strategies, with the given premise of 1041 FCs, 3480 PV modules and 1800 kWh of energy storage. However, this fixed setting ignores the co-optimization of initial capital investment and subsequent energy management operations. Furthermore, it also obscures the differences between various facility combinations, especially when government subsidies are applied to some of them at a micro level. Additionally, most of the existing research has been formulated with a static set of external parameters. Therefore, this paper is inspired to relax the assumption of asset configuration and focus on the stochastic perspective. The well-established Benders decomposition algorithm is adopted here for solving large-scale stochastic optimization problems [37]. Instead of tackling all decision variables and constraints at the same time, it decentralizes the overall problem into multiple subproblems by iteration, which alleviates the computational diﬃculty and improves algorithm efficiency. A variety of its successful applications are found in the energy fields [38].

# Innovations and contributions

The main contributions of this research are as follows:

- The typical driving characteristics associated with ELCVs, as distinct from passenger vehicles, are more fully described and considered in this study than previously.

- One basic and three integrated business models with a mix of facilities are analyzed within the same framework for the first time, including FCs, RCs, PV panels, and new/second-life battery storage systems.

- Distinct from previous studies of pre-specified asset configurations, the optimal installation configuration, energy management and investment feasibility are formulated together, thereby revealing the enhanced benefits of integrated facilities.

- Considering the various uncertainties, a general two-stage stochastic model for charging infrastructure investment and daily operational planning is established, in which a Benders decomposition algorithm is adopted to obtain the optimal solutions.

# Paper organization

The reminder of this paper is structured as follows: In Section 2, the problem description is introduced. Section 3 explains the proposed model formulation. Section 4 presents the input data and stochastic settings. In Section 5, the optimal numerical results are compared followed by the main conclusions and policy implications in Section 6.

# Research framework

The schematic illustration of the charging station is shown in Figure 2. The optimal decisions include two stages: the initial asset configuration and the consequent operational planning, which are closely intertwined. In the investment stage, confronted with physical space constraints, the investor evaluates a range of chargers and related energy resources, such as PV panels, new and second-life battery storage systems. Additionally, as for the regular contracts signed with van distribution companies, such as flower and parcel deliveries, their daily charging demands can be predicted based on the historical data. In the scheduling stage, the charging station is assumed to have a grid connection via a licensed electricity retailer, offering time-of-use consumption and export tariffs. The energy management of the facilities is in the charge of a specialized company, such as an aggregator or an energy service company. With the transactional interface between electric vans and the grid, the aggregator can not only bid for a certain amount of electricity, but also offer ancillary services and achieve the V2G potential. It will also plan the charging and discharging decisions, including arranging the parking time window, selecting the charger type, and specifying the power delivery quantities. Furthermore, it is assumed that these operational strategies are in response to the day-ahead electricity prices rather than the more volatile spot prices. Given the predominance of day-ahead electricity wholesale markets and the routine of commercial delivery routes, a standard daily optimization based upon day ahead information is likely to be preferred in the investment analysis.

Figure 2. Schematic illustration of the charging station

The charging station consisting only of charging facilities such as FCs and RCs is regarded as the basic business model. Since the bi-directional electricity exchange is generally conducted with relatively small rated power capacities [39], it is assumed that only FCs come with V2G function. Considering the technical feasibility and practical application, three additional integrated business models are proposed and correspond to the following specifications.

- S1-basic chargers only, that is, the electricity is procured solely from the power grid.

- S2-integration with chargers and PV panels, which supplies solar energy as an additional source of electricity on the basis of S1.

- S3-integration with chargers, PV panels and new battery storage system, which provides the ability to capture electricity at one time for later use on the basis of S2.

- S4-integration with chargers, PV panels and second-life battery storage system. Compared with S3, it offers another cost-effective but technically less efficient energy storage solution.

# Methodology

# Notation

The notations are defined and summarized in Table 2.

Table 2. Nomenclature and notations

| Category | Symbol | Description | Symbol | Description |
| --- | --- | --- | --- | --- |
| Indices |  | Index of electric vans |  | Index of hours in a day |
|  |  | Index of scenarios |  |  |
| Superscript |  | Fast charger |  | Rapid charger |
|  |  | PV system |  | Battery system |
|  |  | Power grid |  | Electric van |
|  |  | Parking lot |  | Parking space |
|  |  | Charging process |  | Discharging process |
|  |  | Arrival time of electric van |  | Departure time of electric van |
|  |  | V2G mode |  | G2V mode |
|  |  | Day-ahead electricity wholesale market |  | Ancillary services market |
|  |  | Charging service contract |  |  |
| Parameters |  | Unit annualized capital cost |  | Unit annual operating expenditure |
|  |  | Cost of power trading activities |  | Income of power trading activities |
|  |  | Share of stored energy to battery capacity |  | Occurrence number of scenarios in a year |
|  |  | Area of the parking space for a van |  | Area conversion ratio |
|  |  | Quantity of parking spaces |  | Rated power of the charger |
|  |  | Electricity tariff |  | Solar generation per kW of PV system |
|  |  | Battery capacity |  | Maximum capacity of the battery system |
|  |  | Unit degradation cost |  |  |
|  |  | Input and output efficiency |  | Charging and discharging rate |
| Variables |  | Installed configuration |  | Power amount |
|  |  | Available energy stored |  | Binary status of charger connection |
|  |  | Binary status of mode selection |  | Binary status of charging and discharging |

# Model formulations

Due to space limitation, only the specification of S4 is described in detail. The others are special cases with relaxation of the relevant features. The objective function is to minimize the total cost of the investment and operations over all scenarios. The first three lines of Eq. (1) denote the capital costs and operating expenses of charging facilities. Specifically, the first-stage variables include the installed number of FCs and RCs, and the installed capacity of PV and battery system. As expressed by Eqs. (2) and (3), they are constrained by the number and area of parking spaces. The coverage period is set to 1 year and the annualized capital cost is adopted, which is widely used in the investment analysis of facilities with different service life.

|  | (1) |
| --- | --- |
|  | (2) |
|  | (3) |
|  | (4) |

The last line of Eq. (1) is associated with the second-stage variables, which can be interpreted as the cost and income of power trading activities. With a further formula expansion, Eq. (5) represents the expenditure of procuring electricity from grid, which is determined by the amount of power injected from grid to the battery system and the electric vans. Since the V2G mode accelerates the battery degradation, the additional cost is introduced as Eqs. (6) and (7) [40]. Regarding the income, the first source is the sale of solar generation. Ancillary services provided by V2G mode also bring extra income and is reflected by Eq. (9). As shown in Eq. (10), the third part comes from the provision of the charging service to fleet owners, which mainly depends on the charging demand and retail price. Furthermore,  represents the occurrence number of each typical scenario and will be further explained in Section 4.

|  | (5) |
| --- | --- |
|  | (6) |
|  | (7) |
|  | (8) |
|  | (9) |
|  | (10) |

Related constraints for the second-stage variables are described as below. Eq. (11) is to ensure the real-time balance between the supply and demand of solar generation. As for the battery system, Eq. (12) defines the state of available energy over time. Since the capacity of battery is finite, it should be prevented from being charged to an extremely high level or discharged after the stored energy runs out. Therefore, the lower and upper limits of SOC are applied in Eq. (13), which defines the percentage level of available energy relative to battery capacity. Similar to previous work [41], the initial value for the first hour is assigned as Eq. (14). As for Eqs. (15) and (16), the constraints are also imposed on the charging and discharging rate by the symbol of  and , respectively. Furthermore, to incentivize the green electricity export, the V2G payment is only for the battery power that does not arise from the grid purchases [42]. With  and  as the subdivisions, Eqs. (17) and (18) ensure that the exported electricity does not exceed their corresponding original source.

|  | (11) |
| --- | --- |
|  | (12) |
|  | (13) |
|  | (14) |
|  | (15) |
|  | (16) |
|  | (17) |
|  | (18) |

With basically the same working principles as the battery system, electric vans are subject to similar constraints. Eq. (21) is to assign the initial SOC level of the vans upon their arrival, which is featured by fleet driving behavior and will be described in Section 4. As represented by Eq. (22), the aggregator should ensure that when the van leaves, the available stored electricity reaches the level required by the fleet owner. The maximum current flow of G2V activity and V2G activity is defined as Eqs. (25) - (27).

|  | (19) |
| --- | --- |
|  | (20) |
|  | (21) |
|  | (22) |
|  | (23) |
|  | (24) |
|  | (25) |
|  | (26) |
|  | (27) |

There are also binary variables in the second stage, illustrating whether a certain mode is being adopted. Specifically,  is labeled as 0 or 1 in accordance with status that the van  is being charged through a FC or not.  is also defined in this way for RC. As expressed by Eq. (28), only one charger can be selected. Since the charging service can only be completed during a van’s stay in the parking lot, the charging status value in other time periods will be set as 0, as shown in Eqs. (29) and (30). Furthermore, Eqs. (31) and (32) denote that the number of vans which are being charged should not exceed the total number of chargers. As for Eqs. (33) - (35), the right-hand sides are in a unified form of multiplying the binary status with a fixed input parameter. This is to ensure that the power exchanges between vans and other facilities are allowed only if a certain charger is connected. Taking Eq. (33) for instance, when , it can be seen that the sum of charged electricity will not exceed the battery capacity. But this constraint is actually redundant as there are more stringent constraints, such as Eq. (20). However, when , all the variables on the left-hand side will be forced to zero automatically.

|  | (28) |
| --- | --- |
|  | (29) |
|  | (30) |
|  | (31) |
|  | (32) |
|  | (33) |
|  | (34) |
|  | (35) |

With the same logic mentioned in the last paragraph, related constrains for the remaining three sets of binary variables are expressed as follows.  and  display the status of G2V mode and V2G mode.  and  represent the charging and discharging operation of battery system. Similarly,  and are the binary indicators for electric vans.

|  | (36) |
| --- | --- |
|  | (37) |
|  | (38) |
|  | (39) |
|  | (40) |
|  | (41) |
|  | (42) |
|  | (43) |
|  | (44) |
|  | (45) |
|  | (46) |
|  | (47) |
|  | (48) |

# Benders decomposition algorithm

A classical Benders decomposition-based method is adopted here to solve the solutions. Without loss of generality, this mixed-integer problem can be represented as Eq. (49).

|  | (49) |
| --- | --- |

By classifying the integer and binary variables into the master problem, only continuous variables remain in the subproblem to ensure the strong duality [38]. As expressed below, the problem is programmed using General Algebraic Modeling System (GAMS) with CPLEX solver.

| Steps of Benders decomposition algorithm |  |
| --- | --- |
| 1: | Initialize the integer variables, binary variables, lower bound and upper bound |
| 2: | while do |
| 3: | Solve the subproblem |
| 4: | if subproblem is unbounded then |
| 5: | Get unbounded ray and add feasibility cut to master problem |
| 6: | else |
| 7: | Get extreme point and add optimality cut to master problem |
| 8: | Update the upper bound |
| 9: | end if |
| 10: | Solve the mater problem and update the lower bound |
| 11: | end while |

# Simulation settings

As a pioneer in decarbonizing the transportation sector, London is identified as one of the global “EV capitals” [43], which includes 25 metropolitan cities with the most EV registrations in 2020. With the forecast of a tenfold increase in the EV sales by 2025, the roll out of charging facilities needs to be accelerated in London [44]. In addition, the UK has been widely regarded as one of the world’s leaders in liberalization of power generation and retail. Its deregulated electricity market not only contributes to the formation of flexible pricing mechanism, but also provides aggregators with rich service options and revenue streams. This is further helpful to analyze the enhanced benefits of public charging stations from a market-based perspective. Therefore, the above formulations are applied to a case study based upon the London context, which aims to provide insights for more countries that are launching the power sector reform and addressing the availability of charging infrastructure. Three kinds of uncertainties are detailed in Section 4.1: namely, day-ahead electricity prices, solar generation, and the driving characteristics of ELCV fleet. The other input parameters are shown in Section 4.2.

# Uncertainty characterization

# Day-ahead wholesale prices

Whilst the charging station will pay and receive payments based upon arrangements with its retailer, these tariffs are assumed to be wholesale-linked to reflect time of day variations. Similar to Ketterer [45], an AR-GARCH model is proposed. In Eq. (50), the day-ahead electricity price  is the dependent variable. The explanatory variables include the day-ahead demand forecast , the generation forecasts of solar  and wind , natural gas price  and carbon emission price . The dummy variable  correspond to spring, summer, autumn and winter.  refer to sunny, rainy, cloudy and snowy days.  and  stand for a constant term and hourly sequence. The residual series , the conditional variance  and the long-run variance  are in the form of Eqs. (51) and (52).

|  | (50) |
| --- | --- |
|  | (51) |
|  | (52) |

The relevant British historical data from 24 February, 2015 through 23 February, 2020 were obtained from Elexon, Intercontinental Exchange and European Energy Exchange. With the check of Akaike Information Criterion and Ljung-Box Q-test statistics, the specification of AR (3)-GARCH (1,1) model is established. According to Table 3, except for snowy days in spring  and snowy days winter , other dummy variables are statistically significant, which provides a basis for scenario selection .

Table 3. Results of AR–GARCH model

| Mean equation |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Quantitative variables |  |  |  | Dummy variables |  |  |  |  |  |  |  |  |
|  | -5.4690*** |  | 0.0003*** |  |  |  | 0.8718*** |  | -1.5925*** |  |  | -1.5150*** |
|  | 0.9137*** |  | -0.0005*** |  | 0.9885*** |  | 0.9524*** |  | -1.5925*** |  |  | -1.2342*** |
|  | -0.2181*** |  | -0.0002*** |  | 0.4632*** |  | 0.6892*** |  | -1.3659*** |  |  | -1.8352*** |
|  | -0.0472*** |  | 0.2556*** |  | 0.0447 |  |  |  |  |  |  | -0.3310 |
|  |  |  | 0.1551*** |  |  |  |  |  |  |  |  |  |
| Variance equation |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7.9895*** |  | 0.3893*** |  | 0.2607*** |  |  |  |  |  |  |  |
| Fitness evaluation |  |  |  |  |  |  |  |  |  |  |  |  |
| Adjusted R-squared |  |  | 0.7727 |  | P value of Chi-square in ARCH-LM test |  |  |  |  | 0.2142 |  |  |
| AIC |  |  | 5.9260 |  | Mean absolute percentage error (MAPE) |  |  |  |  | 11.36% |  |  |

Note: 1. The declared dummy variables are one less than the classified values of  and  to avoid multicollinearity. The constant  defaults to the case of sunny days in spring .

2. Confidence level is denoted by * for 90%, ** for 95% and *** for 99%.

The forecasts of  are shown in Figure 3. And the probability of each typical scenario is assumed to be its occurrence frequency  throughout the historical data.

Figure 3. Forecast of day-ahead wholesale electricity prices

In the UK, the Smart Export Guarantee has been introduced since 2020 to replace Feed-in Tariff, which obligates retailers to offer and pay tariffs for renewable generation exported to the grid. For instance, Octopus Energy establishes the Agile Outgoing Tariff associated with day-ahead wholesale price , line losses, etc. [46]. As for the consumption price, it designs the Agile Tariff which is also dynamically tied to  [47]. Thus, with its published calculation model, a consistent set of consumption and export price pairs at the retail level can be obtained (see Figure 4).

Figure 4. Consumption and export electricity prices

# Solar generation

Based on the historical data mentioned in 4.1.1, the unit generation of PV system  can be obtained by dividing the predicted solar generation by the total installed capacity. As shown in Figure 5, it is intuitive that the generation is the highest on a sunny day in summer.

Figure 5. Forecast of solar generation per kW of PV system

# Charging demand

In addition to the supply side, the charging station aggregator is also confronted with uncertainties on the demand side, which requires estimation from historic driving information. Due to its large sample size and easy access, the latest 2017 database under the van category from National Household Travel Survey [48] is analyzed. Histograms in Figure 6 suggest that most vans start to be plugged in between 16 to 21 o'clock. Their common mileage is about 50–100 km, and nearly 30% of them travel more than 100 km per day. The kernel density and lognormal distribution are adopted to simulate the probability density function (PDF) and cumulative distribution function (CDF) of  and , respectively.

Figure 6. PDF and CDF of charging start time and daily driving distance

Similar to previous research [49], the fleets are supposed to set out with fully charged batteries, and are parked at the charging station once the delivery task is completed. Given the interference of a large number of random variables, such as charging start time and charging end time, it is usually difficult to obtain the numerical values of the varying outcomes [50]. With focuses on constantly repeating random samples to achieve certain results, Monte Carlo simulation makes probabilistic valuation possible with regard to the volatility of relevant factors [51]. Therefore, it is a suitable research tool for predicting charging demand here. With the steps presented in Figure 7, a Monte Carlo algorithm is implemented.

Figure 7. Flowchart of proposed Monte Carlo algorithm

It is assumed that ,  and [52]. Considering the available capacity of the local distribution network [53], the fleet size  is presumed to range from 10 to 80 vehicles. With , the charging load curves can be obtained. After adopting the k-medoid method to calculate the distance between each curve at each hour, the highest expected case is selected as the representative input parameter of this paper (see Figure 8).

Figure 8. Charging demand in a day with different fleet size

# Local data and parameters

With a parking space of 15 m2,  and  are set as 1.25 and 0.15 [32]. The annual rental fee across London is about 55 £/m2 [54], of which 2% would be attributable to the operating and maintenance (O&M) costs. To fully identify a feasible region for the model, is set as the fleet size multiplied by the capacity of each electric van. According to the main commercial products,  and  are assumed to be 7 kW and 50 kW [52]. With the grid reinforcement service, their connection costs are approximately discounted as 142 £ and 285 £ [53]. The input/output efficiency of the PV inverter, FC and RC are respectively set at 88%, 85% and 85% [30]. Cost parameters are shown in Table 4. In terms of technical performance parameters, Table 5 summarizes the conversion loss between electrical energy and chemical energy, the allowable amount of transferred electricity and the extreme limits of stored electricity relative to battery capacity per unit time. Considering the additional cycles caused by V2G operation, the battery degradation penalty undertaken by the charging station aggregator is also listed below.

Table 4. Cost parameter list

|  | Annualized capital cost | Annual O&M cost | Source |
| --- | --- | --- | --- |
| PV system | 55 £/kW | 11 £/kW | [29,40,55] |
| New battery system | 40 £/kWh | 8 £/kWh |  |
| Second-life battery system | 22 £/kWh | 4.4 £/kWh |  |
| FC | 535 £ | 100 £ | [12,56] |
| RC | 5194 £ | 300 £ |  |

Note: 1. , where capital cost covers procurement and installation of the facility itself and its hardware components. The residual value is 30% of the initial value, and the discount rate is set as 7.0% [11].

2. The annual O&M cost includes maintenance inspection, data collection and warranty insurance.

Table 5. Technical performance parameter list

|  | New battery | Second-life battery | Electric van | Source |
| --- | --- | --- | --- | --- |
| Charge and discharge efficiency | 90% | 85% | 90% | [28,41] |
| Maximum charge and discharge rate | 85% | 85% | 85% |  |
| Minimum SOC level | 5% | 10% | 5% |  |
| Maximum SOC level | 95% | 90% | 95% |  |
| Degradation cost caused by V2G | 1.65 p/kWh | 1.7 p/kWh | 1.6 p/kWh | [39] |

# Results

# Facility configurations

In the aspect of facility configuration, once the solar generation system is introduced, the optimal decision is to make full use of all available space in the parking lot, that is, to reach the upper limit of the installable capacity. For example, when the fleet size is 50, the optimal specifications of PV panels in S2 to S4 are all 31.5 kW. Regarding the energy storage, the low price of second-life batteries helps them gain the edge over the new ones, despite their inferior charging and discharging efficiency. Thus, the optimal battery capacity in S4 is usually larger than that in S3. For instance, in the case of 50 vans, they are correspondingly 33 kWh and 99 kWh for S3 and S4 respectively.

As shown in Figure 9, FCs account for about 62.11%, which is slightly higher than RCs. Besides, for small fleets with less than 50 vans, the optimal balance between FCs and RCs stays the same and simply increases pro-rata with the fleet size. But with larger fleets, the balance becomes more nuanced. Furthermore, the integrated business models make better use of the available space in the parking lot. Taking the case of 80 vehicles as an example, the total charger installations of S3 and S4 increase by 13.33% based on S1.

Figure 9. Installation number of chargers in different business models

# Investment viability and sensitive analysis of subsidies

The indicator of annual return on investment (ROI) is used to measure the investment viability and expressed as:

|  | (53) |
| --- | --- |

As shown in Figure 10, with the expansion of fleet size, the overall ROI shows benefits of scale. Specifically, with a fleet size of more than 50 vans, all these four business modes can achieve ROI ratios greater than 1 without government subsidies. The benefits of synthesis with PV and batteries are progressive and compared with S1, the ROI of S2, S3, S4 increases by 1.74%, 4.58%, and 5.39% on average.

Figure 10. ROI of the charging station in different business models

A fleet size of 50 vans is taken as an example here to explore the impact of government subsidies. Currently, the UK grant towards each charger is 350 £ regardless of the charger type [14]. With the general warranty period of 10 years [39], it is annualized as 50 £. A more ambitious financial incentive is studied, which can be interpreted as a decline in market prices due to technological advances. FCs and RCs are assumed to be subsidized in the range of 0-100 £ in steps of 10 £ (see Figure 11). Although the ceiling of the subsidy has been set relatively high, the optimal installation number of chargers remains the same. In other words, the grant can only be reflected as a reduction in the investment cost, rather than any other additional revenue source triggered by the changes in facility deployment. Therefore, the increase in the subsidy amount linearly promotes the growth of ROI. In view of the existing UK subsidy level (50 £ for both FCs and RCs), the basic chargers-only business model S1 is loss-making. However, S1 can achieve ROI>1 under more generous financial support, such as a differentiated subsidy policy of 50 £ for FCs and 100 £ for RCs, or an undifferentiated subsidy policy of 70 £ for both FCs and RCs.

Figure 11. Impact of subsidy amounts on the ROI of different business models

# Energy management strategies

A fleet size of 50 vans is taken as an example for the analysis here. By summarizing the results of each typical day, the box-plot diagram of Figure 12 shows the comparison of the various power flow volumes. And Figure 13 illustrates the differences in the average daily power flow profile. Its positive quadrants are defined as the power flow between various facilities within the charging site, while the negative parts represent the electricity exported to the grid. The charging demands, outlined by the black lines, are filled by three sources: electricity purchased from grid, solar generation, and discharged power from battery system.

Figure 12. Power flow volumes for one day in different business models

Figure 13. Average power flow profiles over one day in different business models

In S1, the completion of charging task completely relies on external power imports. Despite the lack of storage system in this case, the connection with FCs allows excess electricity stored in van batteries to be sold back to the grid. From Figure 12, it can be seen that this export volume is relatively small compared with other integrated business models. On the one hand, the parking time window is generally not long, thus making it difficult to discover the arbitrage opportunities within a given period. On the other hand, the existence of battery degradation cost also weakens the economic attractiveness of V2G activities.

In S2, solar generation emerges as another additional power source, but the electricity imported from the grid still dominates, which is around 1100 kWh in S1 and S2 as shown in Figure 12. Firstly, this is because the area of the parking space is limited and the PV panel capacity that can be installed is not large. Secondly, as the peak of solar radiation appears before the peak of charging demand, a large part of the PV output cannot be consumed nor stored in time. Trading surplus solar power with the grid seems to be the desirable option, accounting for 93.36% of the total generation. Another subtle distinction is that the average power export from the fleet to grid is around 85 kWh, implying a slight increase of 5.72% based on S1.

In S3 and S4, the utilization of battery system enriches the choice of energy management strategies. As mentioned previously, the optimal battery capacity in S4 is larger than that in S3 due to the lower costs of second-life batteries. Therefore, S4 has greater capacity and time flexibility for purchasing electricity at low prices in advance. From Figure 13, it can be observed that the stored electricity in the battery has become another source for the charging demand. As indicated in Figure 12, there is an obvious drop in the direct import from grid in S3 and S4. Furthermore, the total amount of power exported from the fleet in S3 and S4 is 49.06% and 134.84% higher than that in S1. In addition, almost all solar generation is still sold directly back to the grid instead of being charged into the battery system to extend the holding time. And although the electricity exchange can be realized in both directions, only the power discharged from storage battery to the van battery exists, because the reverse one requires multiple charging and discharging losses, which is uneconomical.

# Carbon intensity

With the hourly carbon intensity data of the British electricity system in each season [57], the carbon emissions over these 12 scenarios can be obtained. Consistent with Figure 13, the positive quadrants in Figure 14 also indicate the carbon emissions caused by the net import power from the grid. The average daily emissions of S1, S2, S3 and S4 are 201.16 kg, 184.79 kg, 187.30 kg and 201.86 kg, respectively. Therefore, the emission factors per unit mileage of electric vans are 39.61g CO2/km, 36.39 gCO2/km, 36.88 gCO2/km, and 39.75 gCO2/km. The emission of newly registered diesel vans in the European Union reached 158.10CO2/km in 2018 [1]. It can be seen that based on the power generation structure of the Britain, the adoption of electric vans is more conducive to achieving emission reduction goals. In particular, the integration with PV system in S2 cuts down the carbon emissions by 8.13% compared with the basic chargers-only business model S1. Considering the actual site area, the upper limit of the PV system is bounded in this research. If this restriction is relaxed, eg with remote connections, the reduction effect could be more significant with a larger installed capacity. S4 does not, however, present better performance on emissions. This is because in addition to the carbon emission intensity, the total emission levels also depend on the net electricity import. In S4, the electricity that export from the van fleets back to the grid actually still needs to be fully charged at the charging station. And there is more charging demand around 20:00 (see Figure 13), which coincides with the period of high carbon emission intensity in the wholesale market.

Figure 14. Carbon emissions for one day in different business models

# Usage of chargers

In order to further analyze the role played by FCs and RCs in different business models, the V2G or G2V usage patterns adopted over the illustrative day is displayed in Figure 15.

Figure 15. Average daily usage of chargers in different business models

In S1 and S2, FCs are mainly responsible for one-way electricity exchange from grid to the fleet. And the occurrence frequency of G2V pattern is basically consistent with the trend of the demand curve (see Figure 8). Additionally, the consumption price and export price in Figure 4 are at a relatively high level around 17:00-20:00, which is designed to relieve the intense pressure on electricity supply. This period is also the main parking time window for vans, which provides price arbitrage opportunities for the charging station aggregator. However, V2G pattern is not favored in these two cases with the adoption rate of 13.86% and 14.56%.

In S3, the integration with battery storage system also affects the role of different types of chargers. Regarding the usage of FCs, the adoption rate of V2G pattern has been enhanced to 20.83%. And as indicated in Figure 12, most of the export power comes from the van batteries themselves. This means that the actual charging demand is greater than the amount of power consumed by the fleet before its arrival. In fact, RCs serve as the supplementary infrastructure to V2G activities. The higher power rating enables them to complete the charging task within a specified time. During the period from 20:00 to 24:00, 53.19% of the G2V connection is implemented by RCs, which is 12.16% higher than that in S1.

The introduction of larger capacity of battery storage system in S4 promotes the installation number of FCs to reach 8, 60% more than S2. By storing back-up power purchased at low prices, more power from the van batteries can be sold to the grid, especially when export prices are at their peak. As for the usage of FCs, the proportion of selecting V2G mode reaches 24.83%, which is 79.08% more than S1. Similarly, the advantages of higher charging speed brought by RCs are also substantial. Its usage frequency reaches 18.17%, which shows an increase of 13.77% compared with S1.

# Conclusions and policy implications

Regarding the public charging stations for electric vans, a basic and three integrated business models are compared, which integrate solar power generation and new/second-life battery storage system with the basic charging facilities. Considering the uncertainties of electricity tariff and solar generation, the optimal infrastructure investment and operational planning has been formulated as a two-stage stochastic optimization model. Based upon a representative case study of a London, the followings results are concluded:

- The optimal facility configuration for charging is nonadditive in composition and nonlinear in scale. There are additional benefits in integrating chargers with PV and storage. Overall, in the integrated business models, the charger installations could be increased by up to 17.06%, and the carbon intensity could be reduced by up to 8.13%.

- Without subsidies, the basic business model can offer a positive investment return only with relatively large fleet size. However, in the integrated business models, the breakeven capacity is reduced from 60 to 40. The return on investment and charger installations could be increased by up to 5.39%.

- With the nondiscriminatory grant annualized as 50 £, the public charging station in the basic business model still suffers losses. A differentiated subsidy policy may be more helpful to achieve a transition from negative to positive return on investment, such as 50 £ for fast chargers and 100 £ for rapid chargers.

- The integration with other facilities makes fast chargers undertake more V2G electricity export. Rapid chargers have the advantage of higher charging speed and mainly play a supporting role for fast chargers, ensuring that the charging task is completed in time. Compared with the basic business model, the V2G pattern adoption rate of fast chargers and the usage frequency of rapid chargers could be increased by up to 52.38% and 2.82% in the integrated business models.

Notwithstanding that many of the parametric assumptions made in the model are subject to variations over time and different local contexts, our results provide following general policy implications:

- It is evident that the widespread development of basic charging stations is financially challenging without public subsidies. Improved incentives from central and local governments will help, but these resources are quite likely to be constrained. With the substantial potential to reduce the public spending, the integrated business models should be promoted as a preferred policy option.

- Planning consents for the charging facilities should be encouraged. In practice, it is likely that the major fleet operators of delivery services, or the commercial vehicle manufacturers themselves, will become stakeholders to some extent in these charging facilities. Through vertical integration with delivery operations and the supply chain for second-life EV batteries, further improvements in the project returns can be made.

- V2G is found to be the major contributor of the revenue stream in the integrated business models. Evidently more attractive export electricity prices could improve the profitability. Indeed the modelling proposed here may be conservative in that respect, since an export tariff is assumed to be offered by a local retailer. If the operations of the facility are undertaken by a market-wide aggregator, there may be greater access to other revenue streams such as reserve, balancing and flexibility services.

# Acknowledgements

The authors gratefully acknowledge the support provided by National Natural Science Foundation of China (No. 71974197 and No. 71774171), Major Program of National Social Science Foundation of China (No. 21ZDA030) and China Scholarship Council. Also, the authors would like to acknowledge the data provided by the Nissan Energy Team.

# References

[1]	European Environment Agency. Average CO2 emissions from newly registered motor vehicles in Europe 2019. https://www.eea.europa.eu/data-and-maps/indicators/average-co2-emissions-from-motor-vehicles/assessment-1.

[2]	Wen Z, Li H, Zhang X, Chi Kin Lee J, Xu C. Low‐carbon policy options and scenario analysis on CO2 mitigation potential in China’s transportation sector. Greenhouse Gases: Science and Technology 2017;7:40–52.

[3]	Lee JCK, Wen Z. Pathways for greening the supply of rare earth elements in China. Nature Sustainability 2018;1:598–605.

[4]	Schmidt M, Staudt P, Weinhardt C. Decision support and strategies for the electrification of commercial fleets. Transportation Research Part D: Transport and Environment 2021;97:102894.

[5]	International Energy Agency. Global EV Outlook 2020 2020. https://www.iea.org/reports/global-ev-outlook-2020.

[6]	Bryden TS, Hilton G, Cruden A, Holton T. Electric vehicle fast charging station usage and power requirements. Energy 2018;152:322–32.

[7]	Li Y, Zhang Q, Liu B, McLellan B, Gao Y, Tang Y. Substitution effect of New-Energy Vehicle Credit Program and Corporate Average Fuel Consumption Regulation for Green-car Subsidy. Energy 2018;152:223–36.

[8]	Shi X, Pan J, Wang H, Cai H. Battery electric vehicles: What is the minimum range required? Energy 2019;166:352–8.

[9]	International Council on Clean Transportation. Emerging best practices for EV charging infrastructure 2017. https://theicct.org/publications/emerging-best-practices-electric-vehicle-charging-infrastructure.

[10]	Kavianipour M, Fakhrmoosavi F, Singh H, Ghamami M, Zockaie A, Ouyang Y, et al. Electric vehicle fast charging infrastructure planning in urban networks considering daily travel and charging behavior. Transportation Research Part D: Transport and Environment 2021;93:102769.

[11]	Figueiredo R, Nunes P, Brito MC. The feasibility of solar parking lots for electric vehicles. Energy 2017;140:1182–97.

[12]	London Borough of Haringey. Planning advice on electric vehicle charging points 2020. https://www.haringey.gov.uk/parking-roads-and-travel/travel/electric-vehicles/electric-vehicles-charging-points/planning-advice-electric-vehicle-charging-points.

[13]	Zap-Map. Map of charging points for electric car drivers in UK 2019. https://www.zap-map.com/live/.

[14]	Office for Low Emission Vehicles. Grant schemes for electric vehicle charging infrastructure 2020. https://www.gov.uk/government/news/update-on-the-infrastructure-grants-schemes.

[15]	Pasaoglu G, Fiorello D, Martino A, Zani L, Zubaryeva A, Thiel C. Travel patterns and the potential use of electric cars–Results from a direct survey in six European countries. Technological Forecasting and Social Change 2014;87:51–9.

[16]	Mouli GC, Bauer P, Zeman M. System design for a solar powered electric vehicle charging station for workplaces. Applied Energy 2016;168:434–43.

[17]	Neyestani N, Damavandi MY, Shafie-Khah M, Contreras J, Catalão JP. Allocation of plug-in vehicles’ parking lots in distribution systems considering network-constrained objectives. IEEE Transactions on Power Systems 2014;30:2643–56.

[18]	Shafie-Khah M, Heydarian-Forushani E, Osório GJ, Gil FA, Aghaei J, Barani M, et al. Optimal behavior of electric vehicle parking lots as demand response aggregation agents. IEEE Transactions on Smart Grid 2015;7:2654–65.

[19]	Shafie-Khah M, Siano P, Fitiwi DZ, Mahmoudi N, Catalão JP. An innovative two-level model for electric vehicle parking lots in distribution systems with renewable energy. IEEE Transactions on Smart Grid 2017;9:1506–20.

[20]	Barfod MB, Kaplan S, Frenzel I, Klauenberg J. COPE-SMARTER–A decision support system for analysing the challenges, opportunities and policy initiatives: A case study of electric commercial vehicles market diffusion in Denmark. Research in Transportation Economics 2016;55:3–11.

[21]	Christensen L, Klauenberg J, Kveiborg O, Rudolph C. Suitability of commercial transport for a shift to electric mobility with Denmark and Germany as use cases. Research in Transportation Economics 2017;64:48–60.

[22]	Sun X-H, Yamamoto T, Morikawa T. Charge timing choice behavior of battery electric vehicle users. Transportation Research Part D: Transport and Environment 2015;37:97–107.

[23]	Sun X-H, Yamamoto T, Morikawa T. Fast-charging station choice behavior among battery electric vehicle users. Transportation Research Part D: Transport and Environment 2016;46:26–39.

[24]	Chiara GD, Cheah L, Azevedo CL, Ben-Akiva ME. A policy-sensitive model of parking choice for commercial vehicles in urban areas. Transportation Science 2020;54.

[25]	Schücking M, Jochem P, Fichtner W, Wollersheim O, Stella K. Charging strategies for economic operations of electric vehicles in commercial applications. Transportation Research Part D: Transport and Environment 2017;51:173–89.

[26]	Nourinejad M, Wenneman A, Habib KN, Roorda MJ. Truck parking in urban areas: Application of choice modelling within traffic microsimulation. Transportation Research Part A: Policy and Practice 2014;64:54–64.

[27]	Novoa L, Brouwer J. Dynamics of an integrated solar photovoltaic and battery storage nanogrid for electric vehicle charging. Journal of Power Sources 2018;399:166–78.

[28]	Tong SJ, Same A, Kootstra MA, Park JW. Off-grid photovoltaic vehicle charge using second life lithium batteries: An experimental and numerical investigation. Applied Energy 2013;104:740–50.

[29]	Han X, Liang Y, Ai Y, Li J. Economic evaluation of a PV combined energy storage charging station based on cost estimation of second-use batteries. Energy 2018;165:326–39.

[30]	Schroeder A, Traber T. The economics of fast charging infrastructure for electric vehicles. Energy Policy 2012;43:136–44.

[31]	Mouli GRC, Kefayati M, Baldick R, Bauer P. Integrated PV charging of EV fleet based on energy prices, V2G, and offer of reserves. IEEE Transactions on Smart Grid 2017;10:1313–25.

[32]	Cardoso G, Stadler M, Bozchalui MC, Sharma R, Marnay C, Barbosa-Póvoa A, et al. Optimal investment and scheduling of distributed energy resources with uncertainty in electric vehicle driving schedules. Energy 2014;64:17–30.

[33]	Funke SÁ, Plötz P, Wietschel M. Invest in fast-charging infrastructure or in longer battery ranges? A cost-efficiency comparison for Germany. Applied Energy 2019;235:888–99.

[34]	Muratori M, Kontou E, Eichman J. Electricity rates for electric vehicle direct current fast charging in the United States. Renewable and Sustainable Energy Reviews 2019;113:109235.

[35]	Muratori M, Elgqvist E, Cutler D, Eichman J, Salisbury S, Fuller Z, et al. Technology solutions to mitigate electricity cost for electric vehicle DC fast charging. Applied Energy 2019;242:415–23.

[36]	Luo Z, He F, Lin X, Wu J, Li M. Joint deployment of charging stations and photovoltaic power plants for electric vehicles. Transportation Research Part D: Transport and Environment 2020;79:102247.

[37]	Magnanti TL, Wong RT. Accelerating Benders decomposition: Algorithmic enhancement and model selection criteria. Operations Research 1981;29:464–84.

[38]	Rahmaniani R, Crainic TG, Gendreau M, Rei W. The Benders decomposition algorithm: A literature review. European Journal of Operational Research 2017;259:801–17.

[39]	Element Energy. Vehicle to Grid Britain 2019. http://www.element-energy.co.uk/publications/.

[40]	Shafie-Khah M, Heydarian-Forushani E, Golshan M, Siano P, Moghaddam M, Sheikh-El-Eslami M, et al. Optimal trading of plug-in electric vehicle aggregation agents in a market environment for sustainability. Applied Energy 2016;162:601–12.

[41]	Tang Y, Zhang Q, Mclellan B, Li H. Study on the impacts of sharing business models on economic performance of distributed PV-Battery systems. Energy 2018;161:544–58.

[42]	Solar Trade Association. Understanding the Smart Export Guarantee 2020. https://www.solar-trade.org.uk/resource-centre/advice-tips-for-households/smart-export-guarantee/.

[43]	International Council on Clean Transportation. Electric vehicle capitals: Accelerating electric mobility in a year of disruption 2021. https://theicct.org/wp-content/uploads/2021/12/EV-Capitals-briefing-v3-dec21.pdf.

[44]	International Council on Clean Transportation. Fulfilling electric vehicle charging infrastructure needs in Greater London and its boroughs 2020. https://theicct.org/wp-content/uploads/2021/06/London-EV-charging-infra-nov2020.pdf.

[45]	Ketterer JC. The impact of wind power generation on the electricity price in Germany. Energy Economics 2014;44:270–80.

[46]	Octopus Energy. Outgoing Octopus 2020. https://octopus.energy/blog/outgoing/.

[47]	Octopus Energy. Electric Car Tariff Solution 2020. https://www.octopusev.com/tariff.

[48]	Federal Highway Administration. National Household Travel Survey 2017. https://nhts.ornl.gov/.

[49]	Shafiee S, Fotuhi-Firuzabad M, Rastegar M. Investigating the impacts of plug-in hybrid electric vehicles on power distribution systems. IEEE Transactions on Smart Grid 2013;4:1351–60.

[50]	Jeon C, Shin J. Long-term renewable energy technology valuation using system dynamics and Monte Carlo simulation: Photovoltaic technology case. Energy 2014;66:447–57.

[51]	Younesi A, Shayeghi H, Safari A, Siano P. Assessing the resilience of multi microgrid based widespread power systems against natural disasters using Monte Carlo Simulation. Energy 2020;207:118220.

[52]	Transport for London. London electric vehicle infrastructure delivery plan 2019. https://www.tfl.gov.uk/modes/driving/electric-vehicles-and-rapid-charging.

[53]	UK EV Supply Equipment Association. A guide on electric vehicle charging and DNO engagement for local authorities 2019. http://ukevse.org.uk/resources/uk-evse-wpd-local-authority-guidance/.

[54]	JustPark. Find parking in seconds 2020. www.justpark.com.

[55]	Tang Y, Zhang Q, Li Y, Li H, Pan X, Mclellan B. The social-economic-environmental impacts of recycling retired EV batteries under reward-penalty mechanism. Applied Energy 2019;251:113313.

[56]	Chen T, Zhang X-P, Wang J, Li J, Wu C, Hu M, et al. A Review on Electric Vehicle Charging Infrastructure Development in the UK. Journal of Modern Power Systems and Clean Energy 2020;8:193–205.

[57]	National Grid ESO. Carbon Intensity 2020. https://www.carbonintensity.org.uk/.